The

CIRCULARITY GAP REPORT

OMAN

Project Annex

Circle Economy

Haarlemmerweg 331, 1051 LH, Amsterdam, the Netherlands

October 13, 2025
v3.1

Authors and contributors: Alex Colloricchio, Irlanda Mora, Sofia Ferrando, Marijana Novak

How to cite this document:

Circle Economy. (2025). The circularity gap report Oman 2025: Project annex (v3.1). Amsterdam: Circle Economy.  Retrieved from: CGRi website

Table of Contents

List of Tables	3

1. How to use this document	3

2. Data and assumptions	5

2.1 Data availability assessment	5

2.2 Data processing and additional information	7

2.2.1  Economy-wide Material Flow Analysis (EW-MFA) data	7

2.2.2 Waste generation and treatment data	8

2.2.3 Extended EW-MFA data tables	10

2.2.4 Circularity Indicator Set approach	12

2.2.5 Comparison of indicators results	13

2.3 Scenario data and assumptions	13

2.3.1 Main cross-system data and assumptions	13

2.3.2 Main data and assumptions per system	14

3. Mapping Tables	17

# List of Tables

Table one: Summary of the main data and sources used in CE’s model and RAG classification……………………...5

Table two: Comparison of GMFD data and Statistical Yearbook on minerals (DE) for year 2023……………………..7

Table three: Mapping table of Be’Ahs waste generation categories and WaW classification…………………….……..8

Table four: Mapping table of Be’Ahs waste treatment  and WaW treatment classification………………………..…….9

Table five: Data sources, assumptions and parameter values for allocation shares between material (mUse %) versus energy use (eUse %) and stock additions (stk%) versus throughput (thr%) materials……………………..10

Table six: Waste flow codes (W-Code) for NACE waste categories as reported by Eurostat, and allocation to main EW-MFA categories and material or energetic use……….…...............................................................................12

Table seven: Comparison of results for the ITCr under different perspectives and assumptions 2023............12

Table eight: Main cross-systems input data and assumptions ...............................................................................13

Table nine: Main housing system input data and assumptions..............................................................................13

Table ten: Main mobility system input data and assumptions................................................................................14

Mapping tables

Mapping table one: Mapping table of Exiobase to Intermediate sectors classification, societal needs and final demand label....................................................................................................................................................................................15

Mapping table two: Mapping table of material flows for Domestic Extraction (DE) between Exiobase

3.8 environmental extension and Eurostat’s EW-MFA classifications…………..……..……………………………………..……19

Mapping table three: Mapping table of countries to continents........……..……...............……………………………..…..25

# 1. How to use this document

This document outlines the data and specific methodological decisions used in the Circularity Gap Report (CGR) Oman.  It is intended for readers familiar with the general methodology document and provides additional information about project-specific choices regarding data collection, reconciliation efforts, and national analyses. It also summarises the methodologies used in the spotlight chapter of the main report.

For more detailed information on the analyses conducted, please refer to the technical annexes of the general methodology document.

# 2. Data and assumptions

To ensure our data accurately reflects the reality of Oman, we collaborated with the Ministry of Economy as facilitators, connecting with various researchers and consulting the Ministry of Economy when needed. Additionally, we conducted desktop research to fill data gaps.

This section outlines the data sources used in the study and evaluates whether the data was adequate to conduct a Circularity Gap assessment for Oman. Unless stated otherwise, the year of study is 2023.

Subsequent sections provide detailed explanations of the data reconciliation steps taken to harmonise international and local data sources into a unified view and comparative evaluations of how different design choices influence ITCr results.

## 2.1 Data availability assessment

Developing a comprehensive SEM analysis at the national level requires combining several separately developed data sources. With the support of local stakeholders, the data is collected through the dissemination of a data availability template. Officially reported data is used where available. Reported data that has been significantly manipulated for use in Circle Economy’s models is documented in Section 1.2. All main datasets used are summarised and evaluated for quality through the Red-Amber-Green (RAG) classification system (Table one). The remaining data gaps or uncertainties are clearly identified to allow for continual improvement of the methodology in the future.

Table one. Summary of the main data and sources used in CE’s model and RAG classification

|  | Reliable data that is up-to-date: annually updated territorial data |
| --- | --- |
|  | Potential inaccuracies: national data (or estimates thereof) scaled to regional level, territorial data not updated or with potential inaccuracies |
|  | Likely inaccuracies: all other data (e.g. proxies) |

| Label | Code | Source | RAG status |
| --- | --- | --- | --- |
| Domestic extraction | DE | GMFD dataset (2023) |  |
| Physical Imports | IMP | GMFD dataset (2023) |  |
| Physical Exports | EXP |  |  |
| Imported secondary materials | SMimp | Circle Economy’s estimation based on the BACI database (2023). HS17 codes matched to CN23 and based on information from Eurostat (see Annex A, Box 2.1) |  |
| Exported secondary materials | SMexp |  |  |
| Waste recycled (domestic secondary materials) | SMdom | Be’ahs Waste Inventory and Oman Environment Authority |  |
| Energetic use | eUse | Calculated based on information in Table five |  |
| Material use | mUse | Eurostat, env_wastrt dataset |  |
| Gross additions to stock | GAS | Calculated based on information in Table five |  |
| Demolition and Discard (D&D) | D&D | Calculated as W_mUse - (mUse-GAS-Wu_mUse) |  |
| Reported waste from energetic use | W_eUse | Assumed to be 0 % |  |
| Reported waste from material use | W_mUse | Assumed to be 100% |  |
| Net additions to stock | NAS | Calculated as GAS - D&D |  |
| Reported End-of-Life waste | EoLr | Be’Ahs Waste Inventory and Oman Environment Authority |  |
| Manure in % of energetic use | Man | Calculated from FAOSTAT number of live animals in combination with metabolic coefficients from UNEP’s EW-MFA manual |  |
| Extractive waste | Ext | Included, but not explicitly quantified. Directly ascribed to IntOut |  |
| Short-lived material use of crops | Crp | Calculated based on information from Mayer et al. (2019) |  |
| Unreported waste from energetic use | Wu_eUse | Approximated with Man |  |
| Unreported waste from material use | Wu_mUse | Assumed to be 45% of crop residues, based on information from Mayer et al. (2019). Crop residues data retrieved from GMFD dataset (2023) |  |
| Total reported waste | Wu | Calculated as Wu_mUse + Wu_eUse |  |
| Total End-of-Life waste | EoLt | Calculated as EoL_r + Wu |  |
| Domestic processed output from energy (emissions) | DPOe | Calculated as eUse-W_eUse -Wu_eUse |  |
| Domestic processed output from materials | DPOw | Calculated as EoL_t - SM_dom |  |
| Domestic processed output | DPO | Calculated as DPOe + DPOw |  |
| Interim outputs | IntOut | Calculated as DPO + Wu |  |
| Balancing items input-side | BIi | All variables are pre-calculated at the net of the balancing items as per extended EW-MFA approach methodology |  |
| Balancing items output-side | BIo |  |  |
| Raw Material Equivalents of Imports | RME_IMP | Weavebase model (see section Environmentally Extended Multi-Regional Input-Output Analysis—Weavebase model in Annex A of the Methodology for Nations document) |  |
| Raw Material Equivalents of Exports | RME_EXP |  |  |
| Deforestation emissions | Def | Commodity-driven deforestation, associated carbon emissions and trade 2001-2022 (2018 and 2022) |  |

## 2.2 Data processing and additional information

### 2.2.1  Economy-wide Material Flow Analysis (EW-MFA) data

For Domestic Extraction, a cross comparison with Oman’s Statistical Yearbook and the GMD for non-metallic minerals and fossil fuels was performed. Table two  compares the extraction of minerals. While minerals within similar categories (e.g., ornamental or building stone, gypsum, limestone) show comparable values, some categories are not present in the yearbook (fertilizer and chemical minerals, clays, etc). This suggests that the scope covered by the Ministry may be narrower. We used IRP data to ensure consistent categorization with other regions in our Multi Regional Input Output model.

Table two.Comparison of GMFD data and Statistical Yearbook on minerals

| Domestic Extraction (000 tonnes) | GMFD (2023) | Statistical Yearbook (2023) |
| --- | --- | --- |
| Ornamental or building stone | 1460 | 1496 |
| Chalk | 0 | - |
| Dolomite | 0 | - |
| Limestone | 16229 | 12630 |
| Fertilizer minerals n.e.c. | 0 | - |
| Chemical minerals n.e.c. | 0 | - |
| Industrial minerals n.e.c | 353 | - |
| Salt | 16 | 7 |
| Gypsum | 12226 | 13253 |
| Structural clays | 3651 | - |
| Specialty clays | 52 | - |
| Industrial sand and gravel | 76 | - |
| Sand gravel and crushed rock for construction | 78114 | - |
| Other non-metallic minerals n.e.c. | 0 | 43053 |
| TOTAL | 112177 | 70440 |

### 2.2.2 Waste generation and treatment data

Waste generation data were obtained from Be’ah’s Waste Inventory, covering Municipal Solid Waste (MSW), Construction and Demolition Waste (C&DW), Industrial Waste (IW), Hazardous Waste, and Medical Waste. Additional data on IW was provided by the Environment Authority; UNITAR data was used for E-waste. All the data sources were then recategorized to align with the What a Waste classification framework used in our analysis. Certain waste categories were excluded either because they are released directly to the environment thus being part of Domestic Processed Output (DPO)—in order to prevent double counting—or because they represent secondary waste generated from treatment processes—which falls outside the EW-MFA system boundaries.

Table three. Mapping table of Be’Ahs waste generation categories and WaW classification

| Waste Category be’ahs Waste Inventory | Waste Category |
| --- | --- |
| Ash from the incinerator. | Exclude (Secondary waste) |
| Asphalt waste | Construction & Demolition Waste |
| Animal Waste | Exclude (Double-counting) |
| Bulky waste | Industrial Waste |
| Contaminated Construction and Demolition | Construction & Demolition Waste |
| C&D Residual | Construction & Demolition Waste |
| Confiscated Waste | Hazardous Waste |
| Dead Animal Waste | Exclude (Double-counting) |
| End Life Tyers | Industrial Waste |
| Glass Waste | Industrial Waste |
| Green waste | Exclude (Double-counting) |
| Marble Waste | Construction & Demolition Waste |
| Municipality solid waste | Municipal Solid Waste |
| Non-Hazardous Industrial Waste | Industrial Waste |
| Plastic Waste | Municipal Solid Waste |
| Shreddered Green Waste | Exclude (Double-counting) |
| Dry Sludge Waste Moisture Level below 20% | Exclude (Secondary waste) |
| Slaughter House Waste | Exclude (Double-counting) |
| Un-Contaminated Construction and Demolition | Construction & Demolition Waste |
| Wood Waste | Industrial Waste |
| Non-Hazardous Medical Waste | Medical Waste |
| Shreddered Bulky Waste | Exclude (Secondary waste) |
| Green waste | Exclude (Double-counting) |
| Shredded Wood Waste | Exclude (Secondary waste) |
| Residual Shredder Area | Exclude (Secondary waste) |

Waste treatment types from Be’ah’s waste inventory were reclassified into the standard categories used in our analysis, following the WaW classification system. Most treatments listed as “storage” were assumed to correspond to “Landfill – Unspecified.” For construction and demolition (C&D) waste, “stockpiling” refers to material that has been processed and meets the required specifications for treatment and recycling, allowing for its subsequent reuse.

Table four. Mapping table of Be’Ahs waste treatment  and WaW treatment classification

| Waste treatment type (from Source) | Category |
| --- | --- |
| Municipal Solid Waste |  |
| LF Cell | Sanitary Landfill |
| Material Recovery Facility | Recycling |
| Green Waste Storage Area | Landfill Unspecified |
| Construction & Demolition Waste |  |
| Stockpilling | Recycling |
| LF Cell | Sanitary Landfill |
| Al Mudhaybi LF | Sanitary Landfill |
| Industry Waste |  |
| Bulky Storage Area | Landfill Unspecified |
| C&D Storage Area | Landfill Unspecified |
| ELT Storage Area | Landfill Unspecified |
| Glass Storage Area | Landfill Unspecified |
| Green Waste Storage Area | Landfill Unspecified |
| LF Cell | Sanitary Landfill |
| Shredder Area(Green/Bulky) | Other Treatment |
| Wood Storage Area | Landfill Unspecified |
| Inert Landfill | Sanitary Landfill |
| Single-lined Landfill | Sanitary Landfill |
| Double-lined Landfill | Sanitary Landfill |
| Solidification | Landfill Unspecified |
| Incineration (Stored) | Incineration |
| Physical & Chemical Treatment (Stored) | Landfill Unspecified |
| Incineration | Incineration |
| Medical Waste |  |
| LF Cell | Sanitary Landfill |
| Incineration | Incineration |
| Autoclave | Sanitary Landfill |
| Hazardous Waste |  |
| LF Cell | Sanitary Landfill |
| C&D Storage Area | Landfill Unspecified |

### 2.2.3 Extended EW-MFA data tables

Below, we document the main sources, assumptions and parameter values used in CE’s EW-MFA model. For an explanation of how these values are used in the model, see Annex A in the general methodology document.

Table five. Data sources, assumptions and parameter values for allocation shares between material (mUse %) versus energy use (eUse %) and stock additions (stk%) versus throughput (thr%) materials

| Main material group | Allocation to material [mUse] or energy use [eUse] | Share | Allocation of material use to additions to stock [Stk] or throughput material [Thr] | Share |
| --- | --- | --- | --- | --- |
|  |  | Year 2023 |  | Year 2023 |
| Biomass | Crops based on FAO food balance sheets, ‘other uses’ in FAO food balance sheets are assigned to material use minus renewable energies (Eurostat, 2017), data cross-checked with Krausmann et al. (2008) Crop residues: 35% to material use and 65% to energy use Wood: Timber allocated to material use, and wood fuel to energetic use | eUse: 96% mUse:4 % | Crops all assigned to energetic use, no additions to stocks Crop residues (straw) assumed 10% stock add based on Haas et al. (2015) Timber (Industrial roundwood) as stock add except for paper production assumed as 10% of roundwood based on Haas et al. (2015) and Krausmann et al. (2008, 2017) | Stk: 67% Thr:33 % |
| Fossil energy carriers | Fossil energy split between energetic and non energetic use according to UNSTAT Energy Balances for Oman. | eUse:75 % mUse:25 % | Fixed coefficients from Mayer et al. (2019) | Stk: 52% Thr:48 % |
| Metals (i.e. metal content) | 100% mUse | eUse: 0% mUse: 100% | Weighted averages based on data for Iron: Wang et al. (2007); Aluminum: Cullen and Allwood (2013); Other metals: (Allwood et al., 2010) | Stk: 79% Thr: 21% |
| Extractive waste (tailings) | Directly ascribed to interim outputs | - | Directly ascribed to interim outputs | - |
| Industrial minerals | Fixed coefficients from Mayer et al. (2019): 100% mUse | eUse: 0% mUse: 100% | mUse Industrial minerals assigned to stocks with the following exceptions: Fertiliser minerals: 100% throughputs Salt: 100% throughputs | Stk: 75 % Thr: 25 % |
| Construction minerals | Fixed coefficients from Mayer et al. (2019) | eUse: 0% mUse: 100% | Directly ascribed to stocks, no wastes and losses during construction are assumed as throughputs | Stk:100 % Thr: 0% |

The conceptual gap between waste statistics and EW-MFA is based on different reporting concepts and classifications, thus requiring allocation and harmonisation. Waste flows are to be allocated to EW-MFA categories based on the main material components of waste flows. This was only feasible at the level of the main material groups, such as biomass, fossil energy carriers, industrial minerals, construction minerals, and metal ores. Table six shows the average proxy material composition of waste used for all countries in the CGR engine, based on Haas et al. (2020). The shares are derived from the composition of the ‘Discard and demolition’ flow for the year 2015. Therefore, they do not take into account waste generated from short-lived products or dissipative losses (labelled as ‘dissipative losses and processing waste’ in the Haas et al. (2020) model), since including these would overestimate the biomass fraction. An exception is made for Metal Ores, where dissipative losses and processing waste are included. This is because extractive waste, such as tailings, is directly assigned to interim outputs, and the metals recorded under ‘Discard and demolition’ are expressed in metal content, while tailings are, at least in theory, already represented in the CGRengine’s waste treatment data. Moreover, because the CGR waste database only includes non-organic waste fractions—with the exception of technical biomass such as timber or paper—all waste is allocated to material use. This assumption leads to an underestimation of energy-related waste, particularly ashes from waste incineration.

Table six. Waste flow codes (W-Code) for NACE waste categories as reported by Eurostat, and allocation to main EW-MFA categories and material or energetic use

| Waste Type | Biomass | Metal Ores | Industrial Minerals | Fossil Fuels | Construction Minerals | Total | Material Use | Energetic Use |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Total Waste | 5.6% | 4.5% | 1.3% | 1.1% | 87.5% | 100% | 100% | 0 |

### 2.2.4 Circularity Indicator Set approach

Indicators can be calculated either on Processed Materials (PM)—based on direct physical flows such as domestic extraction, imports, and exports—or on Processed Raw Materials (PRM), which use Raw Material Equivalents (RME) to reallocate global upstream extraction to the final consumer. While PRM offers a life-cycle perspective by attributing raw material extraction to the point of final consumption, applying the same Circularity Indicator Set to PRM as to PM introduces several conceptual complications.

Direct physical flows describe what happens within the socioeconomic system under study. Because they represent actual material flows crossing the system boundary, their uses and destinations can be traced with reasonable accuracy. However, PRM incorporates extraction that occurs outside the system boundary, in other economies with different production structures. This mismatch of system boundaries requires the introduction of a bridging term: Net Extraction Abroad (NEA), calculated as RMC – DMC. For more information on this see Project Annex A- The Bridging Item issue, in General Methodology;

Integrating NEA into the indicator framework is not straightforward. When NEA is positive, it is added to the Non-Renewable Input (INR) indicator because its final use cannot be reliably traced through foreign supply chains. When NEA is negative, it must be subtracted from INR. This can result in negative INR values, which is conceptually problematic, as an input indicator is not meant to represent net negative flows. These inconsistencies stem directly from extending a PM-based logic—designed for direct flows—to PRM values that include upstream extraction outside the system boundary.

For these reasons, and to more accurately reflect the material dynamics in Oman, the CGR presents the indicator framework using DMC-based indicators, where system boundaries, material uses, and indicator interpretations remain consistent and transparent.

A final point concerns why Net Additions to Stock (NAS) is higher than Raw Material Consumption (RMC) in the Sankey. This is from the same difference in system boundaries between DMC-based and RMC-based indicators. NAS is calculated on a DMC basis, meaning it captures all physical materials that enter and remain within the domestic economy as additions to buildings, infrastructure, and durable assets. RMC, by contrast, reallocates upstream extraction to the final consumer and therefore attributes some of the raw materials embodied in exported products to foreign final demand rather than to Oman. In practice, Oman must construct domestic infrastructure—such as facilities for petroleum extraction and processing—in order to produce goods that are ultimately exported. The materials used for building this infrastructure appear fully in DMC and therefore in NAS, but they are not fully captured in RMC because their upstream extraction is attributed to the economies that consume the exported products. As a result, NAS can exceed RMC, reflecting that part of Oman’s stock accumulation serves production for external final demand, even though the physical structures themselves remain within the domestic system boundary.

### 2.2.5 Comparison of indicators results

Table seven presents a comparison of the ITCr results based on different perspectives and assumptions for 2023. The analysis examines both consumption and production based perspectives under varying the inclusion of the recycling and by-products waste flows. Production based approach is taken into account the locally generated waste while the consumption based approach considers trade flows giving credits to the country collecting waste (incl exports and excl imports) or the country recycling waste (incl imports and excl exports). On the other hand, life-cycle approach is based on RMC, and direct physical approach is based on DMC.

Table seven: Comparison of results for the ITCr under different perspectives and assumptions  2023

| Perspective | Waste accounting | Life-cycle approach* | Direct physical approach** |
| --- | --- | --- | --- |
| Consumption-based | incl. Imports and excl. exports of waste for recycling and by-products |  | 2.0% |
|  | incl. exports and excl. imports of waste for recycling and by-products |  | 1.6% |
| Production-based | incl. Imports and excl. exports of waste for recycling and by-products | 1.1% | 1.3% |
|  | incl. exports and excl. imports of waste for recycling and by-products | 0.9% | 1.0% |

* Processed materials (PM) are based on RMC (consumption-based) and RMI (production-based)

** Processed raw materials (PRM) are based on DMC (consumption-based) and DMI (production-based)

## 2.3 Scenario data and assumptions

### 2.3.1 Main cross-system data and assumptions

Baseline year : 2023

Table eight. Main cross-systems input data and assumptions

|  | RCP2.6 LED (optimistic) | Unmitigated SSP1 (middle) | Unmitigated SSP2 (pessimistic) |
| --- | --- | --- | --- |
| Pop ( population), (million) | Baseline: 5.0 2032: 5.3 2050: 6.2 | Baseline: 5.0 2032: 5.3 2050: 6.2 | Baseline: 5.0 2032: 5.4 2050: 6.5 |
| IC (degree of implementation) | Baseline: 8.8% 2032: 45.7% 2050: 100% | Baseline: 8.8% 2032: 28.6% 2050: 70.6% | Baseline: 8.8% 2032: 23.6% 2050: 53.6% |

### 2.3.2 Main data and assumptions per system

Table nine. Main Housing system input data and assumptions

|  | RCP2.6 LED (optimistic) | Unmitigated SSP1 (middle) | Unmitigated SSP2 (pessimistic) |
| --- | --- | --- | --- |
| Sfut (residential building floor area), m2/pp | Baseline: 26.0 2032: 27.5 2050: 35.6 | Baseline: 26.0 2032: 32.9 2050: 38.9 | Baseline: 26.0 2032: 35.6 2050: 43.6 |
| OR (occupancy rate increase), % | Baseline-2050: 20% | Baseline-2050: 20% | Baseline-2050: 20% |
| LFT (Lifetime extension coefficient), % | Baseline-2050: 89% | Baseline-2050: 89% | Baseline-2050: 89% |
| Share energy-efficient and ZEB building types, % | Baseline: 20% 2032: 60% 2050: 90% | Baseline: 20% 2032: 47% 2050: 70% | Baseline: 20% 2032: 40% 2050: 60% |
| β (design), % | Baseline: 8% downsized, 12 % downsized and lightweighted 2032: 64 % downsized and lightweighted 2050: 85 % downsized and lightweighted | Baseline: 8% downsized, 12 % downsized and lightweighted 2032: 15% downsized, 38 % downsized and lightweighted 2050: 20% downsized, 50 % downsized and lightweighted | Baseline: 8% downsized, 12 % downsized and lightweighted 2032: 15% downsized, 23 % downsized and lightweighted 2050: 20% downsized, 30 % downsized and lightweighted |
| ESP (Energy savings potential of renovation (%) | Baseline: 80% 2032: 80% 2050: 80% | Baseline: 60% 2032: 60% 2050: 60% | Baseline: 30% 2032: 30% 2050: 30% |

Table ten. Main Mobility system input data and assumptions

|  | RCP2.6 LED (optimistic) | Unmitigated SSP1 (middle) | Unmitigated SSP2 (pessimistic) |
| --- | --- | --- | --- |
| Pkm (Person-kilometers travelled) | Baseline: 3195.4 2032: 3794.2 2050: 4000 | Baseline: 3195.4 2032: 5519.7 2050: 6000 | Baseline: 3195.4 2032: 7245.3 2050: 8000 |
| Vkm (Vehicle-kilometers per year) | Baseline: 12000 2032: 12000 2050: 12000 | Baseline: 12000 2032: 12000 2050: 12000 | Baseline: 12000 2032: 12000 2050: 12000 |
| OR (Average vehicle occupancy rate),person/vehicle | Baseline: 1.7 2032: 2.2 2050: 2.3 | Baseline: 1.7 2032: 1.8 2050: 1.8 | Baseline: 1.7 2032: 1.5 2050: 1.5 |
| LFT (Lifetime extension coefficient), % | Baseline-2050: 20% | Baseline-2050: 20% | Baseline-2050: 20% |
| Share of PHEVs, BEVs, FCVs, % | Baseline: 3.3% 2032: 33.3% 2050: 86.5% | Baseline: 3.3% 2032: 6.7% 2050: 19.4% | Baseline: 3.3% 2032: 6.7% 2050: 19.4% |
| DWS (Share of downsized vehicles), % | Baseline: 71% 2032: 88.9% 2050: 91.5% | Baseline: 71% 2032: 79.2% 2050: 80.3% | Baseline: 71% 2032: 69.5% 2050: 69% |
| LWS (Share of lightweight vehicles), % | Baseline: 17.1% 2032: 37% 2050: 60.9% | Baseline: 17.1% 2032: 38% 2050: 61.6% | Baseline: 17.1% 2032: 22.9% 2050: 33.5% |
| CaS ( Share of Pkm delivered by shared cars),% | Baseline: 7.0% 2032: 25.9% 2050: 30% | Baseline: 7.0% 2032: 21.6% 2050: 25% | Baseline: 7.0% 2032: 12.9% 2050: 15% |
| RiS (Share of Pkm delivered by shared rides),% | Baseline: 6.9% 2032: 43.1% 2050: 50% | Baseline: 6.9% 2032: 21.6% 2050: 25% | Baseline: 6.9% 2032: 12.9% 2050: 15% |

Table eleven. Main Nutrition system input data and assumptions

|  | RCP2.6 LED (optimistic) | Unmitigated SSP1 (middle) | Unmitigated SSP2 (pessimistic) |
| --- | --- | --- | --- |
| kcalbase (Average caloric intake (kcal/cap/day)) | Baseline: 3064 2032: 2792 2050: 2250 | Baseline: 3064 2032: 2792 2050: 2250 | Baseline: 3064 2032: 2792 2050: 2250 |
| cveget, cvegan, cmed (Share of population from base to vegetarian and vegan diet (%)) | Baseline: 0 2032: 25% to vegetarian, 25% to vegan 2050: 50% to vegetarian, 50% to vegan | Baseline: 0 2032: 17% to vegetarian, 17% to vegan, 17% to mediterranean 2050: 33% to vegetarian, 33% to vegan, 33% to mediterranean | Baseline: 0 2032: 7% to vegetarian, 7% to vegan, 7% to mediterranean 2050: 17% to vegetarian, 17% to vegan, 17% to mediterranean |
| elimwaste (Share of avoidable waste reduction (%)) | Baseline: 32% 2032: 21% 2050: 10% | Baseline: 32% 2032: 26% 2050: 21% | Baseline: 32% 2032: 29% 2050: 26% |
| xorganic, xseasonal, xlocal(Increase in sustainable agricultural practices (%)) | Organic: 100% Seasonal: 30% Local: 50% | Organic: 100% Seasonal: 30% Local: 50% | Organic: 100% Seasonal: 30% Local: 50% |

Table twelve. Main Manufacturing system input data and assumptions

|  | RCP2.6 LED (optimistic) | Unmitigated SSP1 (middle) | Unmitigated SSP2 (pessimistic) |
| --- | --- | --- | --- |
| Total share of transactions targeted by R-strategies (%) | Industry: 100% Consumer: 50% | Industry: 100% Consumer: 50% | Industry: 100% Consumer: 50% |
| xα,β,γ (Lifetime extension due to R-strategy (%)) | Remanufacturing: 90% Refurbishment: 75% Repair: 30% Reuse: 20% Upgrade: 100% | Remanufacturing: 90% Refurbishment: 75% Repair: 30% Reuse: 20% Upgrade: 100% | Remanufacturing: 90% Refurbishment: 75% Repair: 30% Reuse: 20% Upgrade: 100% |
| yα,β,γ (R-Strategy substitution factor (%)) | Baseline-2050: 20% | Baseline-2050: 20% | Baseline-2050: 20% |
| ktmat (Improvement in fabrication yield (%)) | Vehicles: 10% Machinery: 6% Other products: 9% | Vehicles: 10% Machinery: 6% Other products: 9% | Vehicles: 10% Machinery: 6% Other products: 9% |
| xα,i (Share of new scrap directly reused (%)) | Vehicles: 72% Machinery: 64% Other products: 68% | Vehicles: 72% Machinery: 64% Other products: 68% | Vehicles: 72% Machinery: 64% Other products: 68% |

Table thirteen. Main Lifestyles system input data and assumptions

|  | RCP2.6 LED (optimistic) | Unmitigated SSP1 (middle) | Unmitigated SSP2 (pessimistic) |
| --- | --- | --- | --- |
| xα (Reduction in purchases of clothing (%)) | Baseline-2050: 80% | Baseline-2050: 80% | Baseline-2050: 80% |
| xα (Reduction in purchases of media products and services (%)) | Baseline-2050: 80% | Baseline-2050: 80% | Baseline-2050: 80% |
| xα (Reduction in purchases of household goods (%)) | Baseline-2050: 80% | Baseline-2050: 80% | Baseline-2050: 80% |
| xα(Reduction in market services (%)) | Baseline-2050: 80% | Baseline-2050: 80% | Baseline-2050: 80% |
| yβ (Lifestyle substitution factor (%)) | Baseline-2050: 20% | Baseline-2050: 20% | Baseline-2050: 20% |

# 3. Mapping Tables

Mapping table one: Mapping table of Exiobase to Intermediate sectors classification and societal needs

| Exiobase label | Industry label | Societal needs label |
| --- | --- | --- |
| Cultivation of paddy rice | Agrifood | Agriculture, Food & Beverage |
| Cultivation of wheat | Agrifood | Agriculture, Food & Beverage |
| Cultivation of cereal grains nec | Agrifood | Agriculture, Food & Beverage |
| Cultivation of vegetables, fruit, nuts | Agrifood | Agriculture, Food & Beverage |
| Cultivation of oil seeds | Agrifood | Agriculture, Food & Beverage |
| Cultivation of sugar cane, sugar beet | Agrifood | Agriculture, Food & Beverage |
| Cultivation of plant-based fibers | Agrifood | Agriculture, Food & Beverage |
| Cultivation of crops nec | Agrifood | Agriculture, Food & Beverage |
| Cattle farming | Agrifood | Agriculture, Food & Beverage |
| Pigs farming | Agrifood | Agriculture, Food & Beverage |
| Poultry farming | Agrifood | Agriculture, Food & Beverage |
| Meat animals nec | Agrifood | Agriculture, Food & Beverage |
| Animal products nec | Agrifood | Agriculture, Food & Beverage |
| Raw milk | Agrifood | Agriculture, Food & Beverage |
| Wool, silk-worm cocoons | Agrifood | Consumer & Industrial Manufacturing |
| Manure treatment (conventional), storage and land application | Agrifood | Agriculture, Food & Beverage |
| Manure treatment (biogas), storage and land application | Agrifood | Agriculture, Food & Beverage |
| Forestry, logging and related service activities (02) | Agrifood | Housing, Infrastructure and Utilities |
| Fishing, operating of fish hatcheries and fish farms; service activities incidental to fishing (05) | Agrifood | Agriculture, Food & Beverage |
| Mining of coal and lignite; extraction of peat (10) | Mining and quarrying | Housing, Infrastructure and Utilities |
| Extraction of crude petroleum and services related to crude oil extraction, excluding surveying | Mining and quarrying | Housing, Infrastructure and Utilities |
| Extraction of natural gas and services related to natural gas extraction, excluding surveying | Mining and quarrying | Housing, Infrastructure and Utilities |
| Extraction, liquefaction, and regasification of other petroleum and gaseous materials | Mining and quarrying | Housing, Infrastructure and Utilities |
| Mining of uranium and thorium ores (12) | Mining and quarrying | Housing, Infrastructure and Utilities |
| Mining of iron ores | Mining and quarrying | Consumer & Industrial Manufacturing |
| Mining of copper ores and concentrates | Mining and quarrying | Consumer & Industrial Manufacturing |
| Mining of nickel ores and concentrates | Mining and quarrying | Consumer & Industrial Manufacturing |
| Mining of aluminium ores and concentrates | Mining and quarrying | Consumer & Industrial Manufacturing |
| Mining of precious metal ores and concentrates | Mining and quarrying | Consumer & Industrial Manufacturing |
| Mining of lead, zinc and tin ores and concentrates | Mining and quarrying | Consumer & Industrial Manufacturing |
| Mining of other non-ferrous metal ores and concentrates | Mining and quarrying | Consumer & Industrial Manufacturing |
| Quarrying of stone | Mining and quarrying | Housing, Infrastructure and Utilities |
| Quarrying of sand and clay | Mining and quarrying | Housing, Infrastructure and Utilities |
| Mining of chemical and fertilizer minerals, production of salt, other mining and quarrying n.e.c. | Mining and quarrying | Agriculture, Food & Beverage |
| Processing of meat cattle | Agrifood | Agriculture, Food & Beverage |
| Processing of meat pigs | Agrifood | Agriculture, Food & Beverage |
| Processing of meat poultry | Agrifood | Agriculture, Food & Beverage |
| Production of meat products nec | Agrifood | Agriculture, Food & Beverage |
| Processing vegetable oils and fats | Agrifood | Agriculture, Food & Beverage |
| Processing of dairy products | Agrifood | Agriculture, Food & Beverage |
| Processed rice | Agrifood | Agriculture, Food & Beverage |
| Sugar refining | Agrifood | Agriculture, Food & Beverage |
| Processing of Food products nec | Agrifood | Agriculture, Food & Beverage |
| Manufacture of beverages | Agrifood | Agriculture, Food & Beverage |
| Manufacture of fish products | Agrifood | Agriculture, Food & Beverage |
| Manufacture of tobacco products (16) | Agrifood | Consumer & Industrial Manufacturing |
| Manufacture of textiles (17) | Manufacturing | Consumer & Industrial Manufacturing |
| Manufacture of wearing apparel; dressing and dyeing of fur (18) | Manufacturing | Consumer & Industrial Manufacturing |
| Tanning and dressing of leather; manufacture of luggage, handbags, saddlery, harness and footwear (19) | Manufacturing | Consumer & Industrial Manufacturing |
| Manufacture of wood and of products of wood and cork, except furniture; manufacture of articles of straw and plaiting materials (20) | Manufacturing | Housing, Infrastructure and Utilities |
| Re-processing of secondary wood material into new wood material | Manufacturing | Housing, Infrastructure and Utilities |
| Pulp | Manufacturing | Housing, Infrastructure and Utilities |
| Re-processing of secondary paper into new pulp | Manufacturing | Housing, Infrastructure and Utilities |
| Paper | Manufacturing | Housing, Infrastructure and Utilities |
| Publishing, printing and reproduction of recorded media (22) | Manufacturing | Housing, Infrastructure and Utilities |
| Manufacture of coke oven products | Manufacturing | Housing, Infrastructure and Utilities |
| Petroleum Refinery | Manufacturing | Transportation & Fuels |
| Processing of nuclear fuel | Manufacturing | Housing, Infrastructure and Utilities |
| Plastics, basic | Manufacturing | Consumer & Industrial Manufacturing |
| Re-processing of secondary plastic into new plastic | Manufacturing | Consumer & Industrial Manufacturing |
| N-fertiliser | Manufacturing | Agriculture, Food & Beverage |
| P- and other fertiliser | Manufacturing | Agriculture, Food & Beverage |
| Chemicals nec | Manufacturing | Consumer & Industrial Manufacturing |
| Manufacture of rubber and plastic products (25) | Manufacturing | Consumer & Industrial Manufacturing |
| Manufacture of glass and glass products | Manufacturing | Consumer & Industrial Manufacturing |
| Re-processing of secondary glass into new glass | Manufacturing | Consumer & Industrial Manufacturing |
| Manufacture of ceramic goods | Manufacturing | Consumer & Industrial Manufacturing |
| Manufacture of bricks, tiles and construction products, in baked clay | Built Environment | Housing, Infrastructure and Utilities |
| Manufacture of cement, lime and plaster | Built Environment | Housing, Infrastructure and Utilities |
| Re-processing of ash into clinker | Built Environment | Housing, Infrastructure and Utilities |
| Manufacture of other non-metallic mineral products n.e.c. | Built Environment | Housing, Infrastructure and Utilities |
| Manufacture of basic iron and steel and of ferro-alloys and first products thereof | Manufacturing | Housing, Infrastructure and Utilities |
| Re-processing of secondary steel into new steel | Manufacturing | Housing, Infrastructure and Utilities |
| Precious metals production | Manufacturing | Consumer & Industrial Manufacturing |
| Re-processing of secondary preciuos metals into new preciuos metals | Manufacturing | Consumer & Industrial Manufacturing |
| Aluminium production | Manufacturing | Consumer & Industrial Manufacturing |
| Re-processing of secondary aluminium into new aluminium | Manufacturing | Consumer & Industrial Manufacturing |
| Lead, zinc and tin production | Manufacturing | Housing, Infrastructure and Utilities |
| Re-processing of secondary lead into new lead, zinc and tin | Manufacturing | Housing, Infrastructure and Utilities |
| Copper production | Manufacturing | Housing, Infrastructure and Utilities |
| Re-processing of secondary copper into new copper | Manufacturing | Housing, Infrastructure and Utilities |
| Other non-ferrous metal production | Manufacturing | Housing, Infrastructure and Utilities |
| Re-processing of secondary other non-ferrous metals into new other non-ferrous metals | Manufacturing | Housing, Infrastructure and Utilities |
| Casting of metals | Manufacturing | Consumer & Industrial Manufacturing |
| Manufacture of fabricated metal products, except machinery and equipment (28) | Manufacturing | Consumer & Industrial Manufacturing |
| Manufacture of machinery and equipment n.e.c. (29) | Manufacturing | Consumer & Industrial Manufacturing |
| Manufacture of office machinery and computers (30) | Manufacturing | Consumer & Industrial Manufacturing |
| Manufacture of electrical machinery and apparatus n.e.c. (31) | Manufacturing | Consumer & Industrial Manufacturing |
| Manufacture of radio, television and communication equipment and apparatus (32) | Manufacturing | Housing, Infrastructure and Utilities |
| Manufacture of medical, precision and optical instruments, watches and clocks (33) | Manufacturing | Healthcare & Education Services |
| Manufacture of motor vehicles, trailers and semi-trailers (34) | Manufacturing | Transportation & Fuels |
| Manufacture of other transport equipment (35) | Manufacturing | Transportation & Fuels |
| Manufacture of furniture; manufacturing n.e.c. (36) | Manufacturing | Consumer & Industrial Manufacturing |
| Recycling of waste and scrap | Utilities | Public, Administrative & Commercial Services |
| Recycling of bottles by direct reuse | Utilities | Public, Administrative & Commercial Services |
| Production of electricity by coal | Utilities | Housing, Infrastructure and Utilities |
| Production of electricity by gas | Utilities | Housing, Infrastructure and Utilities |
| Production of electricity by nuclear | Utilities | Housing, Infrastructure and Utilities |
| Production of electricity by hydro | Utilities | Housing, Infrastructure and Utilities |
| Production of electricity by wind | Utilities | Housing, Infrastructure and Utilities |
| Production of electricity by petroleum and other oil derivatives | Utilities | Housing, Infrastructure and Utilities |
| Production of electricity by biomass and waste | Utilities | Housing, Infrastructure and Utilities |
| Production of electricity by solar photovoltaic | Utilities | Housing, Infrastructure and Utilities |
| Production of electricity by solar thermal | Utilities | Housing, Infrastructure and Utilities |
| Production of electricity by tide, wave, ocean | Utilities | Housing, Infrastructure and Utilities |
| Production of electricity by Geothermal | Utilities | Housing, Infrastructure and Utilities |
| Production of electricity nec | Utilities | Housing, Infrastructure and Utilities |
| Transmission of electricity | Utilities | Housing, Infrastructure and Utilities |
| Distribution and trade of electricity | Utilities | Housing, Infrastructure and Utilities |
| Manufacture of gas; distribution of gaseous fuels through mains | Utilities | Housing, Infrastructure and Utilities |
| Steam and hot water supply | Utilities | Housing, Infrastructure and Utilities |
| Collection, purification and distribution of water (41) | Utilities | Housing, Infrastructure and Utilities |
| Construction (45) | Built Environment | Housing, Infrastructure and Utilities |
| Re-processing of secondary construction material into aggregates | Built Environment | Housing, Infrastructure and Utilities |
| Sale, maintenance, repair of motor vehicles, motor vehicles parts, motorcycles, motor cycles parts and accessoiries | Services | Transportation & Fuels |
| Retail sale of automotive fuel | Services | Transportation & Fuels |
| Wholesale trade and commission trade, except of motor vehicles and motorcycles (51) | Services | Public, Administrative & Commercial Services |
| Retail trade, except of motor vehicles and motorcycles; repair of personal and household goods (52) | Services | Public, Administrative & Commercial Services |
| Hotels and restaurants (55) | Services | Public, Administrative & Commercial Services |
| Transport via railways | Services | Transportation & Fuels |
| Other land transport | Services | Transportation & Fuels |
| Transport via pipelines | Utilities | Housing, Infrastructure and Utilities |
| Sea and coastal water transport | Services | Transportation & Fuels |
| Inland water transport | Services | Transportation & Fuels |
| Air transport (62) | Services | Transportation & Fuels |
| Supporting and auxiliary transport activities; activities of travel agencies (63) | Services | Transportation & Fuels |
| Post and telecommunications (64) | Services | Housing, Infrastructure and Utilities |
| Financial intermediation, except insurance and pension funding (65) | Services | Public, Administrative & Commercial Services |
| Insurance and pension funding, except compulsory social security (66) | Services | Public, Administrative & Commercial Services |
| Activities auxiliary to financial intermediation (67) | Services | Public, Administrative & Commercial Services |
| Real estate activities (70) | Services | Housing, Infrastructure and Utilities |
| Renting of machinery and equipment without operator and of personal and household goods (71) | Services | Public, Administrative & Commercial Services |
| Computer and related activities (72) | Services | Public, Administrative & Commercial Services |
| Research and development (73) | Services | Public, Administrative & Commercial Services |
| Other business activities (74) | Services | Public, Administrative & Commercial Services |
| Public administration and defence; compulsory social security (75) | Services | Public, Administrative & Commercial Services |
| Education (80) | Services | Healthcare & Education Services |
| Health and social work (85) | Services | Healthcare & Education Services |
| Incineration of waste: Food | Utilities | Public, Administrative & Commercial Services |
| Incineration of waste: Paper | Utilities | Public, Administrative & Commercial Services |
| Incineration of waste: Plastic | Utilities | Public, Administrative & Commercial Services |
| Incineration of waste: Metals and Inert materials | Utilities | Public, Administrative & Commercial Services |
| Incineration of waste: Textiles | Utilities | Public, Administrative & Commercial Services |
| Incineration of waste: Wood | Utilities | Public, Administrative & Commercial Services |
| Incineration of waste: Oil/Hazardous waste | Utilities | Public, Administrative & Commercial Services |
| Biogasification of food waste, incl. land application | Utilities | Public, Administrative & Commercial Services |
| Biogasification of paper, incl. land application | Utilities | Public, Administrative & Commercial Services |
| Biogasification of sewage slugde, incl. land application | Utilities | Public, Administrative & Commercial Services |
| Composting of food waste, incl. land application | Utilities | Public, Administrative & Commercial Services |
| Composting of paper and wood, incl. land application | Utilities | Public, Administrative & Commercial Services |
| Waste water treatment, food | Utilities | Public, Administrative & Commercial Services |
| Waste water treatment, other | Utilities | Public, Administrative & Commercial Services |
| Landfill of waste: Food | Utilities | Public, Administrative & Commercial Services |
| Landfill of waste: Paper | Utilities | Public, Administrative & Commercial Services |
| Landfill of waste: Plastic | Utilities | Public, Administrative & Commercial Services |
| Landfill of waste: Inert/metal/hazardous | Utilities | Public, Administrative & Commercial Services |
| Landfill of waste: Textiles | Utilities | Public, Administrative & Commercial Services |
| Landfill of waste: Wood | Utilities | Public, Administrative & Commercial Services |
| Activities of membership organisation n.e.c. (91) | Services | Public, Administrative & Commercial Services |
| Recreational, cultural and sporting activities (92) | Services | Healthcare & Education Services |
| Other service activities (93) | Services | Public, Administrative & Commercial Services |
| Private households with employed persons (95) | Services | Public, Administrative & Commercial Services |
| Extra-territorial organizations and bodies | Services | Public, Administrative & Commercial Services |

Mapping table two: Mapping table of material flows for Domestic Extraction (DE) between Exiobase 3.8 environmental extension and Eurostat’s EW-MFA classifications

| Exiobase 3.8 classification | EW-MFA classification 3-4 digits |
| --- | --- |
| Domestic Extraction Used - Crop residues - Feed | Crop residues (used) |
| Domestic Extraction Used - Crop residues - Straw | Crop residues (used) |
| Domestic Extraction Used - Fishery - Aquatic plants | Wild fish catch, aquatic plants and animals, hunting and gathering |
| Domestic Extraction Used - Fishery - Inland waters fish catch | Wild fish catch, aquatic plants and animals, hunting and gathering |
| Domestic Extraction Used - Fishery - Marine fish catch | Wild fish catch, aquatic plants and animals, hunting and gathering |
| Domestic Extraction Used - Fishery - Other (e.g. Aquatic mammals) | Wild fish catch, aquatic plants and animals, hunting and gathering |
| Domestic Extraction Used - Fodder crops - Alfalfa for Forage and Silage | Fodder crops (including biomass harvest from grassland) |
| Domestic Extraction Used - Fodder crops - Beets for Fodder | Fodder crops (including biomass harvest from grassland) |
| Domestic Extraction Used - Fodder crops - Cabbage for Fodder | Fodder crops (including biomass harvest from grassland) |
| Domestic Extraction Used - Fodder crops - Carrots for Fodder | Fodder crops (including biomass harvest from grassland) |
| Domestic Extraction Used - Fodder crops - Clover for Forage and Silage | Fodder crops (including biomass harvest from grassland) |
| Domestic Extraction Used - Fodder crops - Forage Products nec | Fodder crops (including biomass harvest from grassland) |
| Domestic Extraction Used - Fodder crops - Grasses nec for Forage and Silage | Fodder crops (including biomass harvest from grassland) |
| Domestic Extraction Used - Fodder crops - Green Oilseeds for Fodder | Fodder crops (including biomass harvest from grassland) |
| Domestic Extraction Used - Fodder crops - Leguminous nec for forage and Silage | Fodder crops (including biomass harvest from grassland) |
| Domestic Extraction Used - Fodder crops - Maize for Forage and Silage | Fodder crops (including biomass harvest from grassland) |
| Domestic Extraction Used - Fodder crops - Other grasses | Fodder crops (including biomass harvest from grassland) |
| Domestic Extraction Used - Fodder crops - Rye Grass, Forage and Silage | Fodder crops (including biomass harvest from grassland) |
| Domestic Extraction Used - Fodder crops - Sorghum for Forage and Silage | Fodder crops (including biomass harvest from grassland) |
| Domestic Extraction Used - Fodder crops - Swedes for Fodder | Fodder crops (including biomass harvest from grassland) |
| Domestic Extraction Used - Fodder crops - Turnips for Fodder | Fodder crops (including biomass harvest from grassland) |
| Domestic Extraction Used - Fodder crops - Vegetables and Roots, Fodder | Fodder crops (including biomass harvest from grassland) |
| Domestic Extraction Used - Forestry - Coniferous wood - Industrial roundwood | Timber (Industrial roundwood) |
| Domestic Extraction Used - Forestry - Coniferous wood - Wood fuel | Wood fuel and other extraction |
| Domestic Extraction Used - Forestry - Kapok Fruit | Wood fuel and other extraction |
| Domestic Extraction Used - Forestry - Natural Gums | Wood fuel and other extraction |
| Domestic Extraction Used - Forestry - Non-coniferous wood - Industrial roundwood | Timber (Industrial roundwood) |
| Domestic Extraction Used - Forestry - Non-coniferous wood - Wood fuel | Wood fuel and other extraction |
| Domestic Extraction Used - Forestry - Raw materials other than wood | Wood fuel and other extraction |
| Domestic Extraction Used - Fossil Fuels - Anthracite | Hard coal |
| Domestic Extraction Used - Fossil Fuels - Coking coal | Hard coal |
| Domestic Extraction Used - Fossil Fuels - Other bituminous coal | Hard coal |
| Domestic Extraction Used - Fossil Fuels - Sub-bituminous coal | Hard coal |
| Domestic Extraction Used - Fossil Fuels - Lignite/brown coal | Lignite (brown coal) |
| Domestic Extraction Used - Fossil Fuels - Peat | Peat |
| Domestic Extraction Used - Fossil Fuels - Crude oil | Crude oil, condensate and natural gas liquids (NGL) |
| Domestic Extraction Used - Fossil Fuels - Natural gas | Natural gas |
| Domestic Extraction Used - Fossil Fuels - Natural gas liquids | Crude oil, condensate and natural gas liquids (NGL) |
| Domestic Extraction Used - Fossil Fuels - Oil shale and oil sands | Oil shale and tar sands |
| Domestic Extraction Used - Fossil Fuels - Other hydrocarbons | Crude oil, condensate and natural gas liquids (NGL) |
| Domestic Extraction Used - Grazing | Grazed biomass |
| Domestic Extraction Used - Metal Ores - Bauxite and aluminium ores | Bauxite and other aluminium |
| Domestic Extraction Used - Metal Ores - Copper ores | Copper |
| Domestic Extraction Used - Metal Ores - Gold ores | Gold, silver, platinum and other precious metals |
| Domestic Extraction Used - Metal Ores - Iron ores | Iron |
| Domestic Extraction Used - Metal Ores - Lead ores | Lead |
| Domestic Extraction Used - Metal Ores - Nickel ores | Nickel |
| Domestic Extraction Used - Metal Ores - Other non-ferrous metal ores | Other non-ferrous metals |
| Domestic Extraction Used - Metal Ores - PGM ores | Gold, silver, platinum and other precious metals |
| Domestic Extraction Used - Metal Ores - Silver ores | Gold, silver, platinum and other precious metals |
| Domestic Extraction Used - Metal Ores - Tin ores | Tin |
| Domestic Extraction Used - Metal Ores - Uranium and thorium ores | Uranium and thorium |
| Domestic Extraction Used - Metal Ores - Zinc ores | Zinc |
| Domestic Extraction Used - Non-Metallic Minerals - Building stones | Marble, granite, sandstone, porphyry, basalt, other ornamental or building stone (excluding slate) |
| Domestic Extraction Used - Non-Metallic Minerals - Chemical and fertiliser minerals | Chemical and fertiliser minerals |
| Domestic Extraction Used - Non-Metallic Minerals - Clays and kaolin | Clays and kaolin |
| Domestic Extraction Used - Non-Metallic Minerals - Gravel and sand | Sand and gravel |
| Domestic Extraction Used - Non-Metallic Minerals - Limestone, gypsum, chalk, dolomite | Limestone, gypsum, chalk, dolomite |
| Domestic Extraction Used - Non-Metallic Minerals - Other minerals | Other non-metallic minerals n.e.c. |
| Domestic Extraction Used - Non-Metallic Minerals - Salt | Salt |
| Domestic Extraction Used - Non-Metallic Minerals - Slate | Slate |
| Domestic Extraction Used - Primary Crops - Kapokseed in Shell | Other crops (excluding fodder crops) n.e.c. |
| Domestic Extraction Used - Primary Crops - Honey | Other crops (excluding fodder crops) n.e.c. |
| Domestic Extraction Used - Primary Crops - Beeswax | Other crops (excluding fodder crops) n.e.c. |
| Domestic Extraction Used - Primary Crops - Abaca | Other crops (excluding fodder crops) n.e.c. |
| Domestic Extraction Used - Primary Crops - Agave Fibres nes | Fibres |
| Domestic Extraction Used - Primary Crops - Almonds | Nuts |
| Domestic Extraction Used - Primary Crops - Anise, Badian, Fennel | Vegetables |
| Domestic Extraction Used - Primary Crops - Apples | Fruits |
| Domestic Extraction Used - Primary Crops - Apricots | Fruits |
| Domestic Extraction Used - Primary Crops - Arecanuts | Nuts |
| Domestic Extraction Used - Primary Crops - Artichokes | Vegetables |
| Domestic Extraction Used - Primary Crops - Asparagus | Vegetables |
| Domestic Extraction Used - Primary Crops - Avocados | Fruits |
| Domestic Extraction Used - Primary Crops - Bambara beans | Pulses |
| Domestic Extraction Used - Primary Crops - Bananas | Fruits |
| Domestic Extraction Used - Primary Crops - Barley | Cereals |
| Domestic Extraction Used - Primary Crops - Beans, dry | Pulses |
| Domestic Extraction Used - Primary Crops - Beans, green | Vegetables |
| Domestic Extraction Used - Primary Crops - Berries nec | Fruits |
| Domestic Extraction Used - Primary Crops - Blueberries | Fruits |
| Domestic Extraction Used - Primary Crops - Brazil nuts, with shell | Nuts |
| Domestic Extraction Used - Primary Crops - Broad beans, horse beans, dry | Pulses |
| Domestic Extraction Used - Primary Crops - Buckwheat | Cereals |
| Domestic Extraction Used - Primary Crops - Cabbages | Vegetables |
| Domestic Extraction Used - Primary Crops - Canary Seed | Other crops (excluding fodder crops) n.e.c. |
| Domestic Extraction Used - Primary Crops - Carobs | Pulses |
| Domestic Extraction Used - Primary Crops - Carrots | Roots, tubers |
| Domestic Extraction Used - Primary Crops - Cashew nuts, with shell | Nuts |
| Domestic Extraction Used - Primary Crops - Cashew apple | Nuts |
| Domestic Extraction Used - Primary Crops - Cassava | Roots, tubers |
| Domestic Extraction Used - Primary Crops - Cassava leaves | Vegetables |
| Domestic Extraction Used - Primary Crops - Castor oil seed | Oil-bearing crops |
| Domestic Extraction Used - Primary Crops - Cauliflower | Vegetables |
| Domestic Extraction Used - Primary Crops - Cereals nec | Cereals |
| Domestic Extraction Used - Primary Crops - Cherries | Fruits |
| Domestic Extraction Used - Primary Crops - Chestnuts | Nuts |
| Domestic Extraction Used - Primary Crops - Chickpeas | Pulses |
| Domestic Extraction Used - Primary Crops - Chicory Roots | Roots, tubers |
| Domestic Extraction Used - Primary Crops - Chillies and peppers, dry | Other crops (excluding fodder crops) n.e.c. |
| Domestic Extraction Used - Primary Crops - Chillies and peppers, green | Vegetables |
| Domestic Extraction Used - Primary Crops - Cinnamon | Other crops (excluding fodder crops) n.e.c. |
| Domestic Extraction Used - Primary Crops - Citrus Fruit nec | Fruits |
| Domestic Extraction Used - Primary Crops - Cloves | Other crops (excluding fodder crops) n.e.c. |
| Domestic Extraction Used - Primary Crops - Cocoa Beans | Other crops (excluding fodder crops) n.e.c. |
| Domestic Extraction Used - Primary Crops - Coconuts | Other crops (excluding fodder crops) n.e.c. |
| Domestic Extraction Used - Primary Crops - Coffee, Green | Other crops (excluding fodder crops) n.e.c. |
| Domestic Extraction Used - Primary Crops - Coir | Other crops (excluding fodder crops) n.e.c. |
| Domestic Extraction Used - Primary Crops - Cotton Lint | Other crops (excluding fodder crops) n.e.c. |
| Domestic Extraction Used - Primary Crops - Cottonseed | Other crops (excluding fodder crops) n.e.c. |
| Domestic Extraction Used - Primary Crops - Cowpeas, dry | Pulses |
| Domestic Extraction Used - Primary Crops - Cranberries | Fruits |
| Domestic Extraction Used - Primary Crops - Cucumbers and Gherkins | Vegetables |
| Domestic Extraction Used - Primary Crops - Currants | Fruits |
| Domestic Extraction Used - Primary Crops - Dates | Fruits |
| Domestic Extraction Used - Primary Crops - Eggplants | Vegetables |
| Domestic Extraction Used - Primary Crops - Fibre Crops nes | Fibres |
| Domestic Extraction Used - Primary Crops - Figs | Fruits |
| Domestic Extraction Used - Primary Crops - Flax Fibre and Tow | Fibres |
| Domestic Extraction Used - Primary Crops - Fonio | Cereals |
| Domestic Extraction Used - Primary Crops - Fruit Fresh Nes | Fruits |
| Domestic Extraction Used - Primary Crops - Fruit, tropical fresh nes | Fruits |
| Domestic Extraction Used - Primary Crops - Garlic | Other crops (excluding fodder crops) n.e.c. |
| Domestic Extraction Used - Primary Crops - Ginger | Other crops (excluding fodder crops) n.e.c. |
| Domestic Extraction Used - Primary Crops - Gooseberries | Fruits |
| Domestic Extraction Used - Primary Crops - Grapefruit and Pomelos | Fruits |
| Domestic Extraction Used - Primary Crops - Grapes | Fruits |
| Domestic Extraction Used - Primary Crops - Groundnuts in Shell | Nuts |
| Domestic Extraction Used - Primary Crops - Hazelnuts | Nuts |
| Domestic Extraction Used - Primary Crops - Hemp Fibre and Tow | Fibres |
| Domestic Extraction Used - Primary Crops - Hempseed | Other crops (excluding fodder crops) n.e.c. |
| Domestic Extraction Used - Primary Crops - Hops | Other crops (excluding fodder crops) n.e.c. |
| Domestic Extraction Used - Primary Crops - Jojoba Seeds | Oil-bearing crops |
| Domestic Extraction Used - Primary Crops - Jute and Jute-like Fibres | Fibres |
| Domestic Extraction Used - Primary Crops - Kapok Fibre | Fibres |
| Domestic Extraction Used - Primary Crops - Karite Nuts | Nuts |
| Domestic Extraction Used - Primary Crops - Kiwi Fruit | Fruits |
| Domestic Extraction Used - Primary Crops - Kolanuts | Nuts |
| Domestic Extraction Used - Primary Crops - Leeks and other Alliac. Veg. | Vegetables |
| Domestic Extraction Used - Primary Crops - Leguminous vegetables, nes | Pulses |
| Domestic Extraction Used - Primary Crops - Lemons and Limes | Fruits |
| Domestic Extraction Used - Primary Crops - Lentils | Pulses |
| Domestic Extraction Used - Primary Crops - Lettuce | Vegetables |
| Domestic Extraction Used - Primary Crops - Linseed | Oil-bearing crops |
| Domestic Extraction Used - Primary Crops - Lupins | Pulses |
| Domestic Extraction Used - Primary Crops - Maize | Cereals |
| Domestic Extraction Used - Primary Crops - Maize, green | Vegetables |
| Domestic Extraction Used - Primary Crops - Mangoes, mangosteens, guavas | Fruits |
| Domestic Extraction Used - Primary Crops - Mate | Other crops (excluding fodder crops) n.e.c. |
| Domestic Extraction Used - Primary Crops - Melonseed | Nuts |
| Domestic Extraction Used - Primary Crops - Millet | Cereals |
| Domestic Extraction Used - Primary Crops - Mixed Grain | Cereals |
| Domestic Extraction Used - Primary Crops - Mushrooms | Vegetables |
| Domestic Extraction Used - Primary Crops - Mustard Seed | Cereals |
| Domestic Extraction Used - Primary Crops - Natural Rubber | Other crops (excluding fodder crops) n.e.c. |
| Domestic Extraction Used - Primary Crops - Nutmeg, mace and cardamoms | Other crops (excluding fodder crops) n.e.c. |
| Domestic Extraction Used - Primary Crops - Nuts, nes | Nuts |
| Domestic Extraction Used - Primary Crops - Oats | Cereals |
| Domestic Extraction Used - Primary Crops - Oil Palm Fruit | Oil-bearing crops |
| Domestic Extraction Used - Primary Crops - Oilseeds nec | Oil-bearing crops |
| Domestic Extraction Used - Primary Crops - Okra | Vegetables |
| Domestic Extraction Used - Primary Crops - Olives | Oil-bearing crops |
| Domestic Extraction Used - Primary Crops - Onions | Roots, tubers |
| Domestic Extraction Used - Primary Crops - Onions, dry | Roots, tubers |
| Domestic Extraction Used - Primary Crops - Oranges | Fruits |
| Domestic Extraction Used - Primary Crops - Other Bastfibres | Fibres |
| Domestic Extraction Used - Primary Crops - Other melons | Fruits |
| Domestic Extraction Used - Primary Crops - Papayas | Fruits |
| Domestic Extraction Used - Primary Crops - Peaches and Nectarines | Fruits |
| Domestic Extraction Used - Primary Crops - Pears | Fruits |
| Domestic Extraction Used - Primary Crops - Peas, Green | Pulses |
| Domestic Extraction Used - Primary Crops - Peas, dry | Pulses |
| Domestic Extraction Used - Primary Crops - Pepper | Vegetables |
| Domestic Extraction Used - Primary Crops - Peppermint | Vegetables |
| Domestic Extraction Used - Primary Crops - Persimmons | Fruits |
| Domestic Extraction Used - Primary Crops - Pigeon peas | Pulses |
| Domestic Extraction Used - Primary Crops - Pineapples | Fruits |
| Domestic Extraction Used - Primary Crops - Pistachios | Nuts |
| Domestic Extraction Used - Primary Crops - Plantains | Vegetables |
| Domestic Extraction Used - Primary Crops - Plums | Fruits |
| Domestic Extraction Used - Primary Crops - Pome fruit, nes | Fruits |
| Domestic Extraction Used - Primary Crops - Poppy Seed | Cereals |
| Domestic Extraction Used - Primary Crops - Potatoes | Roots, tubers |
| Domestic Extraction Used - Primary Crops - Pulses nec | Pulses |
| Domestic Extraction Used - Primary Crops - Pumpkins, Squash, Gourds | Vegetables |
| Domestic Extraction Used - Primary Crops - Pyrethrum, Dried Flowers | Vegetables |
| Domestic Extraction Used - Primary Crops - Quinces | Fruits |
| Domestic Extraction Used - Primary Crops - Quinoa | Cereals |
| Domestic Extraction Used - Primary Crops - Ramie | Other crops (excluding fodder crops) n.e.c. |
| Domestic Extraction Used - Primary Crops - Rapeseed | Oil-bearing crops |
| Domestic Extraction Used - Primary Crops - Raspberries | Fruits |
| Domestic Extraction Used - Primary Crops - Rice | Cereals |
| Domestic Extraction Used - Primary Crops - Roots and Tubers, nes | Roots, tubers |
| Domestic Extraction Used - Primary Crops - Rye | Cereals |
| Domestic Extraction Used - Primary Crops - Safflower Seed | Oil-bearing crops |
| Domestic Extraction Used - Primary Crops - Sesame Seed | Oil-bearing crops |
| Domestic Extraction Used - Primary Crops - Sisal | Other crops (excluding fodder crops) n.e.c. |
| Domestic Extraction Used - Primary Crops - Sorghum | Cereals |
| Domestic Extraction Used - Primary Crops - Sour Cherries | Fruits |
| Domestic Extraction Used - Primary Crops - Soybeans | Oil-bearing crops |
| Domestic Extraction Used - Primary Crops - Spices nec | Other crops (excluding fodder crops) n.e.c. |
| Domestic Extraction Used - Primary Crops - Spinach | Vegetables |
| Domestic Extraction Used - Primary Crops - Stone Fruit nec, | Fruits |
| Domestic Extraction Used - Primary Crops - Strawberries | Fruits |
| Domestic Extraction Used - Primary Crops - String beans | Pulses |
| Domestic Extraction Used - Primary Crops - Sugar Beets | Sugar crops |
| Domestic Extraction Used - Primary Crops - Sugar Cane | Sugar crops |
| Domestic Extraction Used - Primary Crops - Sugar Crops nes | Sugar crops |
| Domestic Extraction Used - Primary Crops - Sunflower Seed | Oil-bearing crops |
| Domestic Extraction Used - Primary Crops - Sweet Potatoes | Roots, tubers |
| Domestic Extraction Used - Primary Crops - Tallowtree Seeds | Oil-bearing crops |
| Domestic Extraction Used - Primary Crops - Tang. Mand Clement. Satsma | Fruits |
| Domestic Extraction Used - Primary Crops - Taro | Roots, tubers |
| Domestic Extraction Used - Primary Crops - Tea | Vegetables |
| Domestic Extraction Used - Primary Crops - Tea nes | Vegetables |
| Domestic Extraction Used - Primary Crops - Tobacco Leaves | Other crops (excluding fodder crops) n.e.c. |
| Domestic Extraction Used - Primary Crops - Tomatoes | Vegetables |
| Domestic Extraction Used - Primary Crops - Triticale | Cereals |
| Domestic Extraction Used - Primary Crops - Tung Nuts | Oil-bearing crops |
| Domestic Extraction Used - Primary Crops - Vanilla | Vegetables |
| Domestic Extraction Used - Primary Crops - Vegetables Fresh nec | Vegetables |
| Domestic Extraction Used - Primary Crops - Vetches | Pulses |
| Domestic Extraction Used - Primary Crops - Walnuts | Nuts |
| Domestic Extraction Used - Primary Crops - Watermelons | Fruits |
| Domestic Extraction Used - Primary Crops - Wheat | Cereals |
| Domestic Extraction Used - Primary Crops - Yams | Roots, tubers |
| Domestic Extraction Used - Primary Crops - Yautia | Roots, tubers |

Mapping table three: Mapping table of countries to continents

| Country | Continent |
| --- | --- |
| Algeria | Africa |
| Angola | Africa |
| Benin | Africa |
| Botswana | Africa |
| Burkina Faso | Africa |
| Burundi | Africa |
| Cameroon | Africa |
| Cape Verde | Africa |
| Central African Republic | Africa |
| Chad | Africa |
| DR Congo | Africa |
| Djibouti | Africa |
| Eritrea | Africa |
| Swaziland | Africa |
| Ethiopia | Africa |
| Gabon | Africa |
| Ghana | Africa |
| Guinea | Africa |
| Cote d'Ivoire | Africa |
| Kenya | Africa |
| Lesotho | Africa |
| Liberia | Africa |
| Libya | Africa |
| Madagascar | Africa |
| Malawi | Africa |
| Mali | Africa |
| Mauritania | Africa |
| Mauritius | Africa |
| Morocco | Africa |
| Mozambique | Africa |
| Namibia | Africa |
| Niger | Africa |
| Nigeria | Africa |
| Congo | Africa |
| Rwanda | Africa |
| Sao Tome and Principe | Africa |
| Senegal | Africa |
| Seychelles | Africa |
| Sierra Leone | Africa |
| Somalia | Africa |
| South Africa | Africa |
| South Sudan | Africa |
| Sudan | Africa |
| Tanzania | Africa |
| Gambia | Africa |
| Togo | Africa |
| Tunisia | Africa |
| Uganda | Africa |
| Zambia | Africa |
| Zimbabwe | Africa |
| Afghanistan | Asia & Oceania |
| Armenia | Asia & Oceania |
| Australia | Asia & Oceania |
| Azerbaijan | Asia & Oceania |
| Bahrain | Asia & Oceania |
| Bangladesh | Asia & Oceania |
| Bhutan | Asia & Oceania |
| Brunei | Asia & Oceania |
| Cambodia | Asia & Oceania |
| Egypt | Asia & Oceania |
| Fiji | Asia & Oceania |
| French Polynesia | Asia & Oceania |
| Georgia | Asia & Oceania |
| Hong Kong | Asia & Oceania |
| India | Asia & Oceania |
| Indonesia | Asia & Oceania |
| Iran | Asia & Oceania |
| Iraq | Asia & Oceania |
| Israel | Asia & Oceania |
| Japan | Asia & Oceania |
| Jordan | Asia & Oceania |
| Kazakhstan | Asia & Oceania |
| Kuwait | Asia & Oceania |
| Kyrgyzstan | Asia & Oceania |
| Laos | Asia & Oceania |
| Lebanon | Asia & Oceania |
| Macao SAR | Asia & Oceania |
| Malaysia | Asia & Oceania |
| Maldives | Asia & Oceania |
| Mongolia | Asia & Oceania |
| Myanmar | Asia & Oceania |
| Nepal | Asia & Oceania |
| New Caledonia | Asia & Oceania |
| New Zealand | Asia & Oceania |
| North Korea | Asia & Oceania |
| Oman | Asia & Oceania |
| Pakistan | Asia & Oceania |
| Gaza Strip | Asia & Oceania |
| Papua New Guinea | Asia & Oceania |
| China | Asia & Oceania |
| Philippines | Asia & Oceania |
| Qatar | Asia & Oceania |
| Russia | Asia & Oceania |
| Samoa | Asia & Oceania |
| Saudi Arabia | Asia & Oceania |
| Singapore | Asia & Oceania |
| South Korea | Asia & Oceania |
| Sri Lanka | Asia & Oceania |
| Syria | Asia & Oceania |
| Taiwan | Asia & Oceania |
| Tajikistan | Asia & Oceania |
| Thailand | Asia & Oceania |
| Turkmenistan | Asia & Oceania |
| UAE | Asia & Oceania |
| Uzbekistan | Asia & Oceania |
| Vanuatu | Asia & Oceania |
| Viet Nam | Asia & Oceania |
| Yemen | Asia & Oceania |
| Albania | Europe |
| Andorra | Europe |
| Austria | Europe |
| Belarus | Europe |
| Belgium | Europe |
| Bosnia and Herzegovina | Europe |
| Bulgaria | Europe |
| Croatia | Europe |
| Cyprus | Europe |
| Czech Republic | Europe |
| Denmark | Europe |
| Estonia | Europe |
| Finland | Europe |
| France | Europe |
| Germany | Europe |
| Greece | Europe |
| Hungary | Europe |
| Iceland | Europe |
| Ireland | Europe |
| Italy | Europe |
| Latvia | Europe |
| Liechtenstein | Europe |
| Lithuania | Europe |
| Luxembourg | Europe |
| Malta | Europe |
| Moldova | Europe |
| Monaco | Europe |
| Montenegro | Europe |
| Netherlands | Europe |
| Norway | Europe |
| Poland | Europe |
| Portugal | Europe |
| TFYR Macedonia | Europe |
| Romania | Europe |
| San Marino | Europe |
| Serbia | Europe |
| Slovakia | Europe |
| Slovenia | Europe |
| Spain | Europe |
| Sweden | Europe |
| Switzerland | Europe |
| Turkey | Europe |
| Ukraine | Europe |
| UK | Europe |
| Netherlands Antilles | Latin America |
| Antigua | Latin America |
| Argentina | Latin America |
| Aruba | Latin America |
| Barbados | Latin America |
| Belize | Latin America |
| Bermuda | Latin America |
| Bolivia | Latin America |
| Brazil | Latin America |
| British Virgin Islands | Latin America |
| Cayman Islands | Latin America |
| Chile | Latin America |
| Colombia | Latin America |
| Costa Rica | Latin America |
| Cuba | Latin America |
| Dominican Republic | Latin America |
| Ecuador | Latin America |
| El Salvador | Latin America |
| Greenland | Latin America |
| Guatemala | Latin America |
| Guyana | Latin America |
| Haiti | Latin America |
| Honduras | Latin America |
| Jamaica | Latin America |
| Mexico | Latin America |
| Nicaragua | Latin America |
| Panama | Latin America |
| Paraguay | Latin America |
| Peru | Latin America |
| Suriname | Latin America |
| Bahamas | Latin America |
| Trinidad and Tobago | Latin America |
| Uruguay | Latin America |
| Venezuela | Latin America |
| Canada | North America |
| USA | North America |

FAO. (2017). FAOSTAT database [WWW Document]. Retrieved from: Faostat website

Eurostat. (2017). Supply, transformation and consumption of renewable energies [nrg_1007a].

Krausmann, F., Erb, K., Gingrich, S., Lauk, C., & Haberl, H. (2008). Global patterns of socioeconomic biomass flows in the year 2000: A comprehensive assessment of supply, consumption and constraints. Ecological Economics, 65(3), 471-487. doi:10.1016/j.ecolecon.2007.07.012

Krausmann, F., Wiedenhofer, D., Lauk, C., Haas, W., Tanikawa, H., Fishman, T., . . . Haberl, H. (2017). Global socioeconomic material stocks rise 23-fold over the 20th century and require half of annual resource use. Proceedings of the National Academy of Sciences, 114(8), 1880-1885. doi:10.1073/pnas.1613773114

Wang, T., Müller, D. B., & Graedel, T. E. (2007). Forging the anthropogenic iron cycle. Environmental Science & Technology, 41(14), 5120-5129. doi:10.1021/es062761t

Cullen, J. M., & Allwood, J. M. (2013). Mapping the global flow of aluminum: From liquid aluminum to end-use goods. Environmental Science & Technology, 47(7), 3057-3064. doi:10.1021/es304256s

Allwood, J. M., Cullen, J. M., & Milford, R. L. (2010). Options for achieving a 50% cut in industrial carbon emissions by 2050. Environmental Science & Technology, 44(6), 1888-1894. doi:10.1021/es902909k

Oman’s data availability template

https://unstats.un.org/unsd/energystats/dataPortal/ (2022)

IC  applies

Applied only to “efficient” + “ZEB” buildings

Equal across building archetypes, implementation curve apply

Applied only to PHEVs, BEVs, FCVs only

Applied to both privately owned and shared vehicles

UNEP IRP Global Material Flows Database

UNEP IRP Global Material Flows Database

UNEP IRP Global Material Flows Database

https://drive.google.com/file/d/1K4xz8I9YJRwY6pch7jTi9IOlbsItSeHu/view?usp=sharing

Haas, W., Krausmann, F., Wiedenhofer, D., Lauk, C., & Mayer, A. (2020). Spaceship earth's odyssey to a circular economy-a century long perspective. Resources, Conservation and Recycling, 163, 105076.

Implementation curves apply

Share of the reduced transaction value targeted by the R-strategy that is re-spent in repair and maintenance-related services. Applies to all R-strategies.

Implementation curves apply

Implementation curves apply

Implementation curves apply

Implementation curves apply

Implementation curves apply

Share of the reduced transaction value targeted by the Lifestyle change that is re-spent in other products and services. Applies to all Lifestyles.